Parts List and Its Functions:

Microphone Stage (Transmitter - TX)

| Component | Value/Part No. | Function |
| --- | --- | --- |
| Q1 | Electret Microphone | Captures audio signals (voice input) |
| R1 | 1kΩ | Bias resistor for the microphone |
| C1 | 470nF | Coupling capacitor to block DC offset |
| C2 | 10pF | Noise filtering |

Pre-Amplifier Stage (TX)

| Component | Value/Part No. | Function |
| --- | --- | --- |
| U1.1 | NJM5532 | Operational amplifier for signal gain |
| R2, R3, R4 | 47kΩ, 10kΩ, 10kΩ | Voltage divider and biasing resistors |
| C3 | 10pF | Stabilizes the op-amp feedback loop |
| R5 | 1MΩ | Sets input impedance of op-amp |
| C4 | 223pF | High-frequency noise filtering |

RF Oscillator and Modulation Stage (TX)

| Component | Value/Part No. | Function |
| --- | --- | --- |
| Q2, Q3 | BC547 | Transistors for oscillation and amplification |
| R6, R7 | 1MΩ, 22kΩ | Bias resistors for Q2 |
| R8, R9 | 10kΩ, 47kΩ | Bias and stabilization resistors |
| C5 | 1nF | Coupling capacitor |
| L1 | 3uH | Inductor for RF signal generation |
| C6 | 100nF | Decoupling capacitor |
| C7 | 5.6pF | Frequency tuning capacitor |
| C8 | 27pF | Frequency stabilization |
| C9 | 223pF | Filtering capacitor |

Power Supply for Transmitter

| Component | Value/Part No. | Function |
| --- | --- | --- |
| C10 | 16V 470uF | Power supply filtering capacitor |
| Power Supply. | 3-5V | Provides power for the TX circuit |

RF Receiver Stage (RX)

| Component | Value/Part No. | Function |
| --- | --- | --- |
| U3 | Antenna | Captures transmitted RF signals |
| L2 | 1uH | RF signal filtering |
| Q4, Q5 | BC547 | RF amplification and demodulation |
| C11 | 220nF | Coupling capacitor |
| R11, R12 | 10kΩ, 1kΩ | Biasing resistors |
| C13 | 100nF | Filtering capacitor |

Audio Amplification (RX)

| Component | Value/Part No. | Function |
| --- | --- | --- |
| U4 | LM386N | Audio amplifier |
| C14 | 100nF | Decoupling capacitor |
| C16 | 25V 10uF | Power supply filtering |
| C18 | 100nF | Decoupling capacitor |
| C19 | 47nF | High-frequency noise filtering |
| R14 | 10Ω | Bias resistor |
| Speaker | 8Ω | Outputs the received audio signal |

Power Supply for Receiver

| Component | Value/Part No. | Function |
| --- | --- | --- |
| C20 | 25V 100uF | Power filtering capacitor |
| C21 | 25V 220uF | Additional power filtering |
| Power Supply | 3-5V | Provides power for RX circuit |

Transmitter (TX):

- The electret microphone captures voice signals.

- The NJM5532 op-amp amplifies the audio.

- A modulator circuit using transistors (BC547) modulates the audio signal onto an RF carrier.

- The antenna transmits the RF signal.

Receiver (RX):

- The antenna captures RF signals.

- The RF section (BC547 transistors and LC circuits) demodulates the audio.

- The LM386 amplifier amplifies the demodulated signal.

- The speaker outputs the received audio.